# Project Phoenix

# Reinforced Concrete Beam Preliminary Design Report

PRELIMINARY STRUCTURAL DESIGN - ENGINEER REVIEW REQUIRED

# Beam RCB-SUR-DEMO-001 - Suriname sketch-input demonstration beam

Span 5.0 m; section 300 x 500 mm.

Materials: C30/37 concrete and B500B reinforcement.

# 1. Loads and structural analysis

Self weight is 3.75 kN/m. The maximum ULS moment is 114.188 kNm and the maximum ULS shear is 80.156 kN.

# 2. Flexural design

The preliminary bottom reinforcement is 6T12 bottom (As=679 mm2; clear=26 mm). The calculated moment resistance is 123.709 kNm, with utilization 0.923.

# 3. Shear design

The preliminary shear reinforcement is 2-leg T8 stirrups @ 200 mm. The governing resistance is 87.331 kN, with utilization 0.918.

# 4. Serviceability

Estimated deflection is 6.746 mm against a 20.0 mm limit. Estimated crack width is 0.093 mm against a 0.3 mm limit.

# 5. Detailing

Top hanger reinforcement: 2T12 continuous. Design anchorage length: 395 mm. Nominal cover: 40.0 mm.

# 6. Checks

CHK-001 - Statics: PASS - Reactions equal total ULS load.

CHK-002 - Flexure resistance: PASS - MEd/MRd = 0.923

CHK-003 - Neutral axis: PASS - x/(xlim*d) = 0.333

CHK-004 - Minimum reinforcement: PASS - As,prov = 678.6 mm2; As,min = 200.9 mm2

CHK-005 - Maximum reinforcement: PASS - As,prov = 678.6 mm2; As,max = 6000.0 mm2

CHK-006 - Bar fit: PASS - 6T12 bottom (As=679 mm2; clear=26 mm)

CHK-007 - Shear resistance: PASS - VEd/VRd = 0.918

CHK-008 - Shear compression strut: PASS - VEd = 80.2 kN; VRd,max = 538.0 kN

CHK-009 - Deflection: PASS - delta = 6.75 mm; limit = 20.00 mm

CHK-010 - Crack width: PASS - wk = 0.093 mm; limit = 0.300 mm

CHK-011 - Anchorage fit: PASS - lb,design = 395 mm; available = 850 mm

CHK-012 - Support bearing: PASS - bearing utilization = 0.087

CHK-013 - Deep-section review gate: PASS - h = 500 mm; mandatory review threshold = 700 mm

CHK-014 - Fire design boundary: PASS - Fire design is excluded and must be completed separately.

CHK-015 - Professional review gate: PASS - All outputs remain preliminary until signed by a competent structural engineer.

CHK-016 - Standard profile declared: PASS - SUR_STRUCTURAL_PRELIMINARY_2026__PROJECT-SPECIFIED EC2 PRELIMINARY BASIS FOR DEMONSTRATION

# 7. Limitations

- This is a preliminary design engine and not a signed structural calculation.

- The selected standard profile and National Annex parameters must be confirmed for the project.

- Fire resistance, robustness, seismic design, fatigue and accidental actions are outside this v1.0.0 scope.

- Construction tolerances, bar curtailment, laps, couplers, support-zone detailing and actual load paths require engineer review.

- The 2025 Dutch National Annex introduced shear-related changes; deep sections trigger a mandatory review gate.

This document is not approved for permit, tender or execution use until signed by a competent structural engineer.